# GeoPIXE mods needed – open source version

- Fortran

  - Needs to be tested on MAC.

- XQuartz upgrade needed first for MAC display.

  - IDL widget programs often fail to display or function correctly on macOS because of a fundamental mismatch between IDL’s reliance on the X11/XQuartz windowing system and macOS’s native Aqua/Quartz windowing system.

- Plugins

  - XFM

    - Both AS and NSLS are in open source, as source.

    - Sav files moved to “geopixe/off/plugins”, except XFM(AS).

- Bugs

  - Correct_yield

    - If current image is already a mpdam, then the test to form the extensions “dame” comes out .mpdam instead of .damx.

    - Seems to cause infinite loop (executes ‘read_mpdam’ over and over): is trying to find .mpdam file instead of a .damx file in execution of MPDA.

- XFM notes

- Extract spectra

    - Error when YLUT needs to change (data accumulating), demands deleting YLUT

    - While Sort EVT will do the delete and generate a new YLUT

- Local plugins in geopixe/plugins

    - XFM tabs in Regions Table (AS, NSLS)

    - Hotspot plugin.

- Ideas

  - MPDA adapted to target radical changing background shapes rather than yields.

    - The idea is to identify very different/ contrasting composition areas that produce very different background shapes and use these as “endmembers” for MPDA.

    - But full MPDA will refine images, changing composition due to yield correction, hence these contrasting compositions will shift in value, complicating this idea.

    - Can we usefully disable some of this “yield correction”, so that these “endmember” regions do not move in composition, so we still benefit from the mixing of background shapes (in proportion to endmember fraction) while not iterating on composition?

  - How to do background-only-MBDA?

    - Have a normal MPDAM file, where read_da() returns arrays of ‘matrix’ and ‘yield’ over all phases, and set all ‘yield’ the same, so the inverse yield ratios all stay 1.

    - But keep the ‘ncomp’ dimension ‘matrix’, which uses different backgrounds.

    - Perhaps just set all ‘matrix.yield’ equal to ‘original’.

    - Saved all this test as branch “mbda” on GitHub.

  - May be better to use MPDA in a different way instead of adding MBDA

    - Simple method:

      - Target extreme background shape area (e.g. with different hot major elements).

      - Fit each as a pseudo-endmember using the same yields, which means new background shape but the same yields. Make DAMX for each.

      - Correct Yields: Make a matrix that uses the major elements and the composition as measured for these hotspots. Add the DAMX from the fitting. Minerals: Create Phase maps.

      - Apply MPDA sorting using new MPDAM.

      - Does it track the backgrounds better?

      - But perhaps it doesn’t do so well on spectrum overlay element peak heights.

      - But may be suitable for a Wizard tool for automation of this:

        - Have a region file and association extracted spectra.

        - Wizard can fit all using a given PCM, make DAMX for each, add concs of selected elements () to matrix wt%, project onto endmembers, save as Phase maps, save a CORRECT file and generate a MPDAM file. User can simple use it as MPDA.

    - Better method:

      - As above but use more accurate yield calcs that use the measured hot-spot values (perhaps X1.2 of majors to allow for shift after Correction).

      - Does it track the backgrounds, overlays and concentrations better?

  - Ctrl middle mouse hold – to show RGB zoom image instead of normal.

  - Uses current RGB elements, as in RGB window.

    - What if no RGB window open?  Default to some strong elements?

  - Will need 24 bit colours on display Draw.

- New stats for better diagnosis of interference, etc.

  - Image: Need a “goodness of fit” map layer.

    - Is this related to total variance measure (or ratio) per pixel?

  - Spectrum: Use final variance to just Poisson stats ratio to flag parts of spectra at risk

    - Add spectra overlay of relative overlap errors.

  - Detector spectra: add a stat which measures departures from the mean

    - Remove general detector variation with radius from mean

    - Perhaps add this to the ‘detector_map’ window?

    - Really want this as a map that highlights pixels with diffraction.

    - Perhaps add this to ‘da_evt’ to accumulate this stat as a map.

  - DA: Perhaps add error matrix to DA fit output to DAMx files

    - use the off-diagonal measures of which elements suffer interferences more.

    - Add this matrix to the Interelement Corrections window?

- Fitting and DA build wizard

  - Guide through fitting and element selection.

    - Use fitting accuracy and RME Error as a guide to completion.

    - Does it include expected elements too?

  - Guide selection of an average matrix composition for the whole area. Not just a mineral.

- Fix missing rows is SLOW

  - 10s for a stretch of 1000 pixels per element.

- Maia device

  - Why does “maia_device::ylut” not enforce a veto on Y Encoder mode?

  - Did I start working on support for a parallel version?

- Working with small ROI in huge images

  - Extract ROI as region view

    - Can be resorted, spectra extracted from regions, etc.

- Docs

  - Worked examples on Batch Processing and/or Wizard.

- XFM wish list

  - PDF HowTo doc for beamline/ASCI

    - Regions on overview

      - Export table to XFM

      - Export overview image with ALL regions, absolute coordinates.

    - Add to Worked Examples, or a cut-down set for beamline?

- Specific tests

  - Using Demo data and worked examples guide

    - Need to add some “answers” to the guide.

    - Then check against these …

  - Some included basic tests

    - May be able to have an automated test (e.g. PFA result against a PFA test file?).

    - See “unit tests” below.

- Unit test style code and data?

  - As an alternative to testing using the Demo data, for example, need lower-level tests of sub-systems.

    - Will need test data (file/files) and setup parameters (perhaps as a GCF setup file).

  - Tests:

    - Source spectrum and parameters

    - Pink beam spectrum and parameters

    - Spectrum fitting, without/with yield/correct iteration

      - Need to use ‘fit_setup’ GUI to load PCM (and data) and do calculation.

        - Results saved in ‘fit_results’ to PFR file.

        - Needs a recipe of data, PCM w/ yields, detector selection, …

        - May need guide a bit like ‘worked examples’ for test cases.

      - Compare fit to spectrum, conc results, errors, MDLs.

        - Compare fit results by reading PFR file.

        - Is “Refit” an option?

        - Spectrum fit compare would be separate.

        - Veto Compton for PIXE (z1 > 0)

    - DA matrix generation.

      - Needs a recipe of data, PCM w/ yields, detector selection, …

    - Sorting from raw data, non-MPDA at least

      - Need a list of steps to follow, and data files to use.

  - Unit tests

    - Unit test data would need to be included in the ‘geopixe’ distribution

      - Need to be able to construct a long list of tests and run them all

        - Perhaps only after setting the Demo root path.

        - Multiple for each test (e.g. a few yield sample/beam combinations).

        - Maybe from a Wizard calling ‘geopixe_execute’ directly?

        - After manually running each module (e.g. yields), or …

        - Wizard could trigger ‘layer_setup’ to load LCM and run it, etc.

    - May make sense to execute these by the GCF mechanism.

      - Need to add entry points for yields and fitting.

      - Then they can be run from a shell script.

      - Need to log smart test results (minimal if results seem OK).

      - Perhaps detect lack of a .GCF file as argv[0]

        - interpret argv[0] as simply the routine to run.

        - Other arguments are ‘files’ - the two files to compare and ‘output’

    - Or, make a GUI program that performs these tests and then does the compare and displays the results.

      - Could just display output file text in a list window.

      - Yields: compare yields and report or plot % differences.

      - Fit: compare fit and report rms% diff and also iterations, Chi, etc.

      - Images: compare images and display images or % (or absolute) differences.

    - This GUI may be a Wizard

      - Use Wizard mechanisms to trigger various GUIs for imaging, yields, fitting …

      - Yields, fits, images could be ‘pages’ in Wizard, with results in window.

      - Only makes sense for GUI level triggers (some may need to be added).

      - Need to log smart test results (minimal if results seem OK).

    - Execute from a “tests” menu?

- GeoPIXE standalone module

  - Modules for a pipeline:

    - Other pipeline processing modules needed?

      - Export of DAI in some form

        - HDF, binary DMT?

      - Image processing

        - Zero pixel removal, which can be slow.

        - How to make access to the image processing features versatile?

      - Parallel options already in Batch Sort?

        - To follow da_evt to DAI, as in Batch Sort.

        - Image processing (using a template DAI)

        - Save as HTML/PNG or TIF options

        - Export image data?

        - Export as selected RGB images (using Learn file).

        - Perhaps add these as new keywords to ‘da_evt’ in GCF, which get detected (in ‘geopixe_do_command’) and removed from args passed to ‘da_evt’.

      - Export plots – element or RGB

      - Export DA for real-time processing elsewhere (export_DA2.pro)

        - No, better to use “Exp” button.

    - Batch Sort options in Command File approach

      - Workflow triggered, but with added outputs, like with Batch Sort.

  - Support for Unit Tests – see above.

GeoPIXE

  - Bugs

    - Correct Y step current

      - No hour glass

    - File preview

      - Detector?

      - Filters?

    - image table

      - When change conc droplist, need to set selected row back again.

    - If load PCM into Fit-setup, while Cluster image sort happening,

      - Nothing seems to appear in fit-setup.

    - KVS access timeout bug

      - Perhaps have a regular heartbeats that read ‘test’?

      - This should keep connection open.

    - Exclude highlight using Box not working

      - sometimes?

    - sort evt – 3D mode

      - “da_xanes_base” is undefined, when switching to tomo XY mode.

      - in tomo XY scan mode, how do we select the tomo element?

        - tomo is 4D, so don’t select element.

        - should not have XANES element selection on DA tab.

    - “Array …” selector from Select Spectrum window

      - What happens when spectra are not single detector # spectra?

        - e.g. Region spectra using all (most) channels?

      - Need to start with closest match to detector?

        - Is the detector data file named?

  - Smart DA selection

    - Preview elements

      - Load DA matrix lists all elements in fit in DA - done

    - Filter elements

      - Filter DA matrix list to include all elements in a selected PCM?

      - Would need some extra ‘filter’ option on file_requester.

        - Pass a ‘filter_routine’ option.

        - All files get passed to filter routine for true/false to display in list.

      - “Special:” shows passed special filter text

        - e.g. “Filter by PCM element list”.

      - Droplist (like plugins)

        - Auto populate list, as with plugins

          - call with /title for droplist text

        - Start with ‘(none)’ on droplist

        - When selected from list, call routine (“??_special_filter)

          - Pass ‘pspecial’ to allow old settings to return

          - Pop up periodic table for selection

        - Pspecial pointer

          - Pointer to array of pointers, one for each special filter

          - Pointer for “Filter by PCM element list”

            - List of elements (K, L and M separately)

        - How to channel events from periodic table into requester?

          - See “select_periodic”, which does this to ‘source_setup’

  - Preview map algorithm

    - Something better than planes 2-4?

  - Export DA for real-time

    - Export_DA2

      - ASCII version seems to be missing: ca, cb, adc, n_det.

        - But these don’t need to be in the DA export file.

        - They are needed in the real-time processing routine for the current detector setup at runtime.

      - ‘Get_linear’ probably assumes (single) old format vector.

        - To use version 2, will need to calculate linear vectors (for all quadrants?).

        - May need to restrict only to version 1, single vector.

      - Where is ‘da.ecompress’ used? Lund old format?

    - Further work:

      - Does not export the rGamma matrix or Pure arrays. This is needed(?) to properly integrate contributions across an array (using selected channels) for quantitative estimation from regions/spectra, or at least for the spectra overlays.

  - Pileup effects

    - Can we use the sum deficit to modify the model pileup spectrum?

    - Fudged in ‘Spectrum_Mark_Pileup_Image’ at 0.2% fixed for now.

  - Export options for Association to parallel those for Image

  - DA for STIM and RBS fitting

    - See email from HW – DA on fit to edges

  - Image, Association window view range

    - Option to fix range, so it does not change.

  - Image intensity low range box

    - Like the high setting in RGB window, also set low value by number.

  - 3D spectral cube

    - Can we load this into 3D window?

    - dynamically display spectra for points and regions?

  - Colour legend

    - Pop up a colour legend (menu: “Display->Colours->Legend”)

    - resizable.

    - Must be linked to parent image window, so if table changes, update it.

    - Save to image file.

  - File Requester

    - Find

      - Should show found files in the file List?

      - Difficult when in the grouped leaf “123…345”.

  - CUT effects on spectrum overlay

    - Interpolate across cut for spectrum values in fit.

  - Fake elements

    - Need to be able to add a “fake element”, with specified energies, relative intensities and (relative?) widths.

    - Use to “fit” diffraction peaks artefacts that arise from time to time (e.g. UQ Herbarium samples).

    - How to add/edit these – pop up?

    - how/where are these saved? In PCM file?

  - PCA-Clusters

    - Need: include, refine, reject, clear menus – like corr

  - Batch image processing

    - Operation order?

      - Need to think about whether we need to introduce a "priority" in the order of these operations. For example, should we do a global "Correct Zero pixels" before individual median filters? Or should that be the other way round? Should all operations performed on an image be tagged by a sequence number, so we can do them again in the same order?

      - Does the History decode the order OK?

      - How do encode a sequence number in the History info.

  - Wizard general

    - Use new ‘widget_browser’ for rich text options?

    - See ‘test_browser_wizard’ in project test/IDL (GeoPIXE3).

  - Standards Wizard

    - Sequence ends if an error is found – should just skip to next?

    - If a result is posted, then “Process” should not do it again - skip?

  - MPDA work-flow

    - Options?

      - As Batch sort extras,

      - Or start to build MPDA Wizard?

        - May take over much of Batch Sort functionality?

      - Or, just a simple flow-on from a MPDA sort

        - In absence of phases, it does normal sort, correct, project Minerals?

        - And then builds mpdam file and repeats as MPDA sort?

    - Distinguish between

      - Building phase regions, all DA matrix files and a new .correct file.

        - Extract suitable regions and spectra.

        - Fit spectra, with appropriate yield for phase, and make DA matrix.

        - Use “MPDA Master Weights” in DA matrix

        - Construct .correct file.

      - Using an existing .correct file (including DA matrix files) to:

        - Extend Sort Evt behaviour.

  - MPDA issues

    - If not cluster mode, how to do ‘geopixe_do_DA’?

      - Perhaps always build ‘args’ and call ‘geopixe_execute’ as used in ‘geopixe_do_command’.

      - But then can’t run in VM mode!

    - Pt hotspots may appear as only 3% Pt mineral and 97% silicates. But ALL of the Pt in them is from the Pt phase component.

      - Fit using straight Pt works, MPDA rel int poor (Lg too high).

      - What relative intensities should be used?

    - Perhaps distinguish between trace elements and majors in formula?

      - But you weight by phase DA matrix, not element.

  - Correct Yield Wizard

    - How to guide through regions analysis?

      - Define major distinct phases and extract spectra from them.

        - Try to find pixels close to the pure phases.

        - Don’t want them too diluted with non-pure phase contributions.

      - Fit each one to create a DA matrix for each using correct yields.

    - Need a “wt%” tool/help to fill in composition matrix.

      - If define element columns first, then a “get” of a DA matrix row just fills in the wt% for these elements in the table?

        - DA matrix file does NOT have this info.

          - Yield file does, so perhaps save this filename in DA matrix?

          - New version would be a pain for existing files.

        - Or, read_yield when typing in mineral name?

        - OK in COMAT file, as we use yield files there.

          - But do we want yields to override table entries?

      - Or, have a pop-up tool that reports wt% from a selected .yield file?

    - Save .Correct file for image correction.

      - Do we correct images here, or below?

        - Have another tab for image correction and projecting phase maps.

        - Some overlap with first tab of MPDA Wizard?

      - Should have guides, or tests, to ensure phases are reasonable

        - Should not go well above 1.0 before clipping.

        - Should not be too small (Rest too close to 1.0 for most pixels).

          - Exception might be bone-like endmember in tissue ‘Rest’.

        - Build this into existing ‘correct_yields’ as tests and pop-ups.

    - For MPDA, continue on …

      - For DA6, make Master Weighting spectrum from the region spectra.

  - Spawn delays

    - Perhaps keep background processes alive, and then send new commands to them?

      - Avoids ~30 seconds lost launching 10 processes.

      - Send new commands, return replies.

      - Must generate prefix internally for file names, and open new, in loop after shared memory setup.

      - May be fixed with IDL 9.0?

    - What if they die?

      - How to ‘know’ that they are still alive?

      - Use a heart-beat approach?

        - Process loops waiting for new command, sets shared memory flag (count?) – but what about during a long operation?

          - Can ‘worker_progress’ keep it alive?

        - GeoPIXE checks flag (and decrements it) on timer, if zero flag a relaunch for next spawns.

      - If any doubts with them, kill and spawn afresh next time.

    - parallel_client

      - Config

        - Only define shared memory of the segment name is blank or /force

      - Init

        - Only define bridge objects if any are NOT valid

        - Still wait for them to be ready (existing ones straight away)

      - Start

        - Load command and args as before

      - Progress, Results

        - Unchanged

      - Monitor

        - Unchanged?

      - Release, Cancel, Reset

        - Unchanged, but not called until GeoPIXE exits?

    - Cancel from spawn background processes, in parallel_init?

      - Needs to kill all processes and exit?

      - Or is this done above in cleanup after cancel/skip?

  - How to deal with Ga Kb peak in MM spectra

    - Follow Ga K lines from source through filter

      - How are In, Sn filtered?

    - Real source lines

      - Source lines are scattered back from sample, and so ‘appear’ to come from a decaying effective conc of these source elements into the sample. They don’t follow the depth profile of these elements (if they are present in the sample).

      - The rel-int for these lines has been modified by source, windows, air and beam filters; they are not the same as rel-int usually. It really needs a different model integration, with decay with depth as effective conc, and modification of the beam lines rel-int on way out.

      - Then in a real spectrum, there may be BOTH real conc in sample and scattered lines from source, for the same element (e.g., Ga). Need to be able to select, perhaps, or combine?

    - Select source or sample lines?

      - ‘get_lines’ does not use source lines now, but it could, if enabled. Then these could be jammed in for matching Z and shell. Would still follow sample conc with depth, which is wrong, but may give reasonable rel-int anyway.

      - Where to select source lines to take precedence?

        - Must be during yield calculation, not the fit.

        - Could select in ‘get_lines’ called from ‘geo_yield2’.

      - Could simply assume that source lines take priority?

        - Then just jam them in, in ‘get_lines’.

        - Make sure Ga K not filtered out somewhere …

  - Pileup peak widths

    - Could add a ‘sum_spread’ argument to pixe() to broaden sum peaks.

    - In ‘pixe’ set ‘sum’ flag (like ‘comp’) to broaden sum peaks in ‘line’ using a new ‘sum_spread’ parameter. Set this in ‘pixe_fit’ based on ‘sum_deficit’.

    - Try: sum_spread = (1.0 + sum_deficit/100.)^2

    - Test using spectrum: software/data/UQ-iXRF/Loren-July-2021/…

  - file_requester

    - Add controls under dir tree in ‘file_requester’:

      - Alphabet order (default), Latest first

  - Xray fit

    - XRF tab

      - Knob arrow-key inc/dec appears too small?

      - Tests show it only incs/decs 1 integer unit and then no more with more clicks.

    - Compton shift

      - In Ka Compton seems to need a different ‘shift’ to the Sn Ka Compton.

      - (e.g. see 9001049 in Mouse project)

      - How does Compton scale with energy? Are In, Sn calculated separately?

  - Cw_fslider2

    - Under Linux, using the arrow keys on the control knob does not work properly.

    - Perhaps add arrows at ends, like in Windows, and control the small inc/dec directly?

    - Make these inc/dec in ‘scroll’ steps.

  - Add more of the Correction Operations to the Image Operations window list

    - Also, make these available via ‘OnButton_Image_Process_Get’, so they work with “Get” and also triggered from “Apply Digital Filters” in batch sorting.

    - This includes “Correct Zero Pixels”.

    - Possibles:

      - Shift rows

      - Zero pixels

      - Missing rows (only if NOT using region).

  - Edit image charge

    - When no flux, just total charge, from History window

  - Possible Y scale options

    - X^(1./3), X^(1./5), X^(1./10), atan(x)

    - IDL does not have ‘asinh’ or ‘atanh’ functions.

  - Regions/ delete tab:

    - Add right, left arrow to move to next prev.

  - -q1 detect in filenames

    - Also look for -q2, -q3, -q4

    - Perhaps use util routine:  ‘strip_file_keys, /file_m’ more universally?

  - Associations

    - Image load puts pre-smooths back to 3 instead of new default 1.