# Multiphase DA imaging (MPDA)

MPDA is a new approach to quantitative imaging that solves: (i) quantitative per pixel concentrations (equivalent to the older "Correct Yields" matrix method), (ii) better X-ray relative intensities that evolve spatially with composition, and (iii) improved background  shapes that adapt with composition changes.

At present it is experimental and triggered by supplying a new .mpdam file in place of the DA matrix file (.dam for PIXE, .damx for SXRF) on the Sort EVT window DA/Cal tab.  The .mpdam file is a simple text file containing the following lines:

version (-1 for now)

path to 'phase map' .dai file

path to the "Correct Yields" setup .correct file

For example: file 111163-Q2.mpdam in the Demo/Maia/Rock3/mpdam dir:

-1

E:\Demo\Maia\Rock3-partial\mpdam\analysis\111163\111163-phases-Q2.dai

E:\Demo\Maia\Rock3-partial\mpdam\analysis\phases2.correct

To use it, you need the following (later we'll have a Wizard to orchestrate these steps):

- Identify end-members

  - Identify end-members components, and elements that provide a measure of each.

  - Fit regions that sample the end-members as much as possible, using a yield calculation with composition for the end-member, and make a DA matrix for each.

  - Also fit a region that samples a background component that will be used as a remainder component called "Rest".

- Correct images using the "Correct Yields" method

  - Open the "Correct Yields" window and select components, elements that sample each and their matching DA matrices from above, and fill in the composition matrix that relates end-members to their elements.

  - Also select the DA matrix to be used as the remainder "Rest" component.

  - Open the .dai image foe for your sample, and click "Correct" to correct concentrations based on the end-member (phase) proportions. Typically, this correction needs 2-3 iterations (press button again).

d. Save the setup as a .correct file (this path will be referenced from the .mpdam file).

- Project end-member or phase maps

  - Click in "Mineral" to project the end-member or phase maps.

  - Save this as your 'phase map' .dai file (this path will be referenced from the .mpdam file).

- Create your .mpdam file in an editor

  - Use the format above, and the 'phase map' and 'Correct setup' file paths from above.

- Select this .mpdam file in Sort EVT (DA/Cal tab)

  - Change the file filter to "*.*" and select the .mpdam file.

- Start the sort (you can use "cluster" mode for Maia data)

  - Normal GeoPIXE functions work the same from there, but you should see better concentrations in images, and better region spectra overlays.

Warnings:

- Phase maps only work if the data is quantitative so you get realistic phase proportions for each component.